# Essay Questions:

## The most popular wireless Internet access technologies are:

- Wi-Fi

  - Uses radio waves to provide high-speed Internet access

  - Operates in 2.4 GHz and 5 GHz frequency bands

  - Short range (typically up to 100 meters)

  - High bandwidth (up to several Gbps with Wi-Fi 6/6E)

  - Widely available in homes, offices, and public spaces

- Cellular Networks (4G LTE and 5G)

  - Provide mobile broadband access over wide areas

  - 4G LTE offers speeds up to hundreds of Mbps

  - 5G provides ultra-high speeds (potentially over 1 Gbps), low latency, and massive device connectivity

  - Requires cellular subscription

- Fixed Wireless Access (FWA)

  - Uses cellular or proprietary technologies to deliver broadband to fixed locations

  - Suitable for rural or suburban areas

  - Can provide high-speed internet without need for wired infrastructure

- Satellite Internet

  - Provides coverage in remote areas where terrestrial networks are unavailable

  - Higher latency due to signal travel time to/from satellites

  - Speeds have improved with new low Earth orbit (LEO) satellite constellations

Comparison:

  - Wi-Fi offers the highest speeds but limited range

  - Cellular networks provide wide coverage and mobility

  - FWA is cost-effective for rural broadband

  - Satellite can reach the most remote locations but with higher latency

## 2. Circuit-Switched vs. Packet-Switched Networks

- Advantage of circuit-switched networks:

  - Guaranteed bandwidth and consistent performance for the duration of the connection

- Advantages of TDM over FDM in circuit-switched networks:

  - More efficient use of bandwidth

  - Easier to reconfigure and scale

  - Less susceptible to interference between channels

## 3. Delay Components in End-to-End Packet Transmission

Delay components:

- Processing delay (variable)

- Queuing delay (variable)

- Transmission delay (constant for fixed packet size and link capacity)

- Propagation delay (constant for a fixed route)

## 4. The five layers in the Internet protocol stack are:

- Application Layer

  - Responsible for supporting network applications

  - Protocols: HTTP, SMTP, FTP, DNS

- Transport Layer

  - Provides process-to-process data transfer

  - Protocols: TCP, UDP

- Network Layer

  - Moves packets from source to destination

  - Protocols: IP, routing protocols

- Link Layer

  - Handles data transfer between neighboring network elements

  - Protocols: Ethernet, Wi-Fi

- Physical Layer

  - Responsible for transmitting individual bits over the physical medium

  - Deals with electrical, optical, or radio transmission characteristics

Each layer provides services to the layer above it and uses services from the layer below it, creating a modular and flexible network architecture.

# Problems:

## Problem 1:

Circuit switching is the optimal choice for applications transmitting data at a steady rate over extended periods. This method establishes a dedicated communication path, ensuring consistent bandwidth allocation and uninterrupted transmission of N-bit units at regular intervals. Circuit switching provides low latency and predictable performance, making it ideal for real-time data requirements and long-running operations. In contrast, packet switching introduces variable latency and potential congestion, which can compromise the consistency required by steady-rate applications. For scenarios demanding reliable and efficient data flow over prolonged periods, circuit switching offers superior stability and performance compared to packet switching.

## Problem 2:

Suppose there is a 10 Mbps microwave link between a geostationary satellite and its base station on Earth. Every minute the satellite takes a digital photo and sends it to the base station. Assume a propagation speed of 2.4 * 108 meters/sec.

What is the propagation delay of the link?

Propagation time= distance / propagation speed.

= x /				
=?				150ms

What is the bandwidth-delay product, R * dprop?

Bw delay product = tp * BW
=10M * (x) ms		150ms
= ?Kbit		150 kbit

Let x denote the size of the photo. What is the minimum value of x for the microwave link to be continuously transmitting?

Transmission time= 1minutes= 60s
= message size / BW
=x / 10M
X = bit

## Problem 3:

Suppose Host A wants to send a large file to Host B. The path from Host A to Host B has three links of rates R1 = 500 kbps, R2 = 2 Mbps, and R3 = 1 Mbps.

Assuming no other traffic in the network, what is the throughput for the file

transfer?

R1 = 500 kbps

R2 = 2 Mbps

R3 = 1 Mbps

The throughput for the file transfer = min {R1,R2,R3}
=min {500 kbps, 2 Mbps, 1 Mbps}
=500 kbps

Suppose the file is 4 million bytes. Dividing the file size by the throughput, roughly how long will it take to transfer the file to Host B?

The file size= 4 million bytes = 32000000 bits.
Throughput FT = 500 Kbps = 500000 bps
file size / throughput FT 
= 32000000 bits / 500000 bps
= 64 seconds

Repeat (a) and (b), but now with R2 reduced to 100 kbps.

R2 reduced to 100 kbps.

R2=100 kbps , R1 = 500 kbps, and R3 = 1 Mbps

The throughput FT =min {R1, R2, R3} =min {500 kbps, 100 kbps, 1 Mbps} =100 kbps

file size / Throughput FT

= 32000000 bits / 100000 bps

= 320 seconds

## Problem 4:

Suppose users share a 2 Mbps link. Also suppose each user transmits continuously at 1 Mbps when transmitting, but each user transmits only 20 percent of the time.

When circuit switching is used, how many users can be supported?

Link size / transmitting size = 2Mbps / 1Mbps = 2

For the remainder of this problem, suppose packet switching is used.

Why will there be essentially no queuing delay before the link if two or fewer users

transmit at the same time?

Since each user requires 1Mbps when transmitting, if two or fewer users transmit simultaneously, a maximum of 2Mbps will be required. This does not exceed the 2Mbps of available bandwidth in the shared link.

Why will there be a queuing delay if three users transmit at the same time?

If there are 3 users then the 3Mbps needed exceeds the 2 Mbps available, there will be a queueing delay.

Find the probability that a given user is transmitting.

p = 20/100 = .2

Suppose now there are three users. Find the probability that at any given time, all three users are transmitting simultaneously.

=0.008