## Network Layers

Data Plane and Control Plane

- The network layer is present in most network devices, except for basic switches. It can be divided into two main components: The Data Plane and the Control Plane.

## Data Plane

- Operates locally within each router.

Forwarding: Responsible for directing packets to their destination.

Local Decision Making: Makes immediate decisions based on local forwarding tables.

- Each router maintains a local forwarding table that helps it quickly determine where to send incoming packets.

## Control Plane

- Operates across the entire network, not just within individual devices.

Routing: Involves determining the best paths for data to travel across the network.

Non-local Decision Making: Takes into account the entire network's structure and conditions when making routing decisions.

Software Defined Networking (SDN):

- In SDN, the control plane is separated from the data plane.

- A remote controller calculates optimal routing paths and installs forwarding tables in routers, allowing for more flexible and dynamic network management.

## Network Service Model

Threading: The network service model provides threads for managing data flow.

Quality of Service (QoS) Guarantees?

- The network does not guarantee specific QoS levels.

Best Effort Services: The service model operates on a "best effort" basis, meaning it tries to deliver packets without guaranteed performance metrics like bandwidth or latency.

## Routing Architecture

- Routing Management Control: Manages how routing decisions are made and updated.

- Forwarding: The process of sending packets to their destinations based on routing decisions.

- Layer Breakdown:

  - Physical Layer: Handles the actual transmission of raw bits over a physical medium.

  - Link Layer: Responsible for node-to-node data transfer and error detection/correction.

  - Decentralized Switching: Each device makes independent forwarding decisions based on the destination of the packets; a method known as destination-based forwarding.

## Input Port Queuing

- When a network switch or router receives packets, it may get more than it can handle at once. Incoming packets are temporarily held in a queue until they can be processed.

- This helps manage traffic and prevents packets from being lost when too many arrive at once.

## Head-of-Line Blocking (HOL Blocking)

- This happens when the first packet in a queue is being processed, which stops the packets behind it from being sent out, even if they could be processed right away.

- It can slow down the overall system because the next packets are stuck waiting.

## Buffering and Round-Trip Time (RTT)

Buffering: The memory set aside to hold packets when they can't be processed right away.

RTT: The time it takes for a packet to go to its destination and back. Longer distances can mean longer RTTs, so more buffering is needed to avoid losing packets during this delay.

Number of Flows

- This refers to how many separate streams of data (like different applications or users) are trying to communicate at the same time.

Complexity: More flows mean more challenges in managing traffic effectively.

Buffer Management

- Techniques used to handle packets in the queue, like how to use memory, when to drop packets, and how to prioritize which packets to process first.

Packet Dropping

- When the memory (buffer) is full, new packets may be discarded to prevent overflow.

- Can involve random choices or specific strategies for which packets to drop.

Tail Drop

- A simple method of dropping the last packet in the queue when the buffer is full, without considering which packets are most important.

- Priority and Packet Scheduling

- Techniques used to determine which packets get processed first based on their importance.

Types:

First-Come, First-Served (FCFS): Processes packets in the order they arrive. Simple, but can cause delays due to HOL blocking.

Round Robin: Each packet gets a turn in a rotating fashion. Fair, but not always efficient.

Weighted Fair Queuing (WFQ): Allocates more bandwidth to higher-priority packets while still allowing others to go through.

## Path Selection Algorithm

- Methods for finding the best route for data packets to travel through a network. Factors like the shortest or least busy path are considered.

- Algorithms like Dijkstra's and Bellman-Ford help in making these decisions.

## IP Protocols

- The set of rules for addressing and routing packets on a network. It includes how packets are formatted and sent.

Versions: Includes IPv4 and IPv6, which are different formats for these addresses.

## ICMP Protocol

- A protocol used for reporting errors and sending operational messages in a network, like informing when a host is unreachable.

## IP Datagram

- The basic packet of data used in the Internet Protocol.

- It includes a header with source and destination addresses and the actual data being sent.

Fragmentation: Large packets can be broken into smaller pieces to be sent more easily.

## Subnets

Each isolated network is called a subnet.

A subnet is a smaller network within a larger network, designed to improve organization and security.

## CIDR (Classless Inter-Domain Routing)

CIDR is a method for allocating IP addresses and routing Internet Protocol packets that replaced the older system of classful addressing. It was introduced in the 1990s to improve the efficiency of IP address allocation and to enhance the scalability of the Internet.

Functions:

- Address Allocation: CIDR allows for more flexible and efficient allocation of IP address ranges. Instead of being restricted to fixed class sizes (A, B, C), it enables the use of any length subnet prefix.

- Routing Efficiency: CIDR helps reduce the size of routing tables by allowing the aggregation of multiple IP addresses into a single routing entry. This process is known as "route aggregation" or "supernetting."

- Scalability: By avoiding classful addressing limitations, CIDR supports the growth of the Internet, accommodating a larger number of networks and devices.

- How It Works: CIDR uses a notation called CIDR notation, which combines the IP address with a prefix length (192.168.1.0/24). The number after the slash represents the number of bits used for the network portion of the address. This allows for variable-length subnet masking (VLSM).

- Subnet Portion of Addresses: In CIDR, the subnet portion of an address can be of arbitrary length, meaning you can create subnets of various sizes based on actual needs rather than being restricted to predefined classes.

## Classful Addressing

Classful addressing is the original method of IP address allocation where the address space was divided into classes (A, B, C, D, E) based on fixed sizes:

- Class A: Large networks (1.0.0.0 to 126.0.0.0)

- Class B: Medium networks (128.0.0.0 to 191.255.0.0)

- Class C: Small networks (192.0.0.0 to 223.255.255.0)

- Class D: Multicast addresses (224.0.0.0 to 239.255.255.255)

- Class E: Reserved for future use (240.0.0.0 to 255.255.255.255)

## Limitations of Classful Addressing

Wasted IP Addresses: Classful addressing often resulted in a waste of IP addresses, as many organizations would receive more addresses than they needed.

Inefficient Routing: The rigidity of classful boundaries led to larger routing tables, as each class had to be individually recognized and managed by routers.

Scalability Issues: The fixed nature of classes limited the growth and scalability of networks.

- DHCP (Dynamic Host Configuration Protocol)

- DHCP is a protocol that automatically assigns IP addresses to devices within a network.

Client Request: A client device initiates a request for an IP address.

Server Response: The DHCP server receives the request and responds with an available IP address.

Additional Settings:

Along with the IP address, the server provides other essential network settings, such as:

- Subnet mask

- Default gateway

  - Automatic IP Assignment: Eliminates the need for manual IP configuration, simplifying network management.

  - Time Efficiency: Saves time during device configuration, making it easier to onboard new devices.

  - Conflict Reduction: Minimizes the risk of IP address conflicts by managing the allocation process.

Protocol Type:

- Operates using UDP (User Datagram Protocol).

Ports Used:

- Port 67 for the server

- Port 68 for the client

## Client Server interactions

Discover:

- The client sends a DHCPDISCOVER message to locate a DHCP server.

Offer:

- The server replies with a DHCPOFFER message, including an available IP address.

- The client accepts the offer by sending a DHCPREQUEST message back to the server.

Ack:

- The server confirms the allocation with a DHCPACK message.

Dynamic Management:

- The server maintains a pool of IP addresses and tracks allocations to prevent duplicates, ensuring effective and efficient network configuration.

## IP Addresses and Subnetting Simplified

- An IP address is a unique number that identifies a device on a network, allowing it to communicate with other devices.

- Subnetting divides a larger IP address space into smaller parts (subnets) for better organization and management.

How Does a Network Get Its Subnet?

From the ISP: Internet Service Providers (ISPs) have large blocks of IP addresses.

- When a network signs up with an ISP, it receives a portion of this address space.

Creating Subnets: The network can then split this allocated space into smaller subnets for different uses, like separating departments.

## Network Address Translation (NAT)

- NAT is a method used in networking that allows multiple devices on a local network (LAN) to share a single public IP address.

  - It translates private IP addresses within a local network to a public IP address for communication over the internet.

  - This allows multiple devices to connect to the internet using one IP address.

How we get our WAN:

A Wide Area Network (WAN) connection typically comes from an Internet Service Provider (ISP). The ISP provides the public IP address that NAT uses.

How it connects to our LAN:

NAT connects the WAN to the LAN by mapping the private IP addresses of devices in the LAN to the public IP address. This is done through a router that implements NAT.

NAT Routing:

- NAT modifies the source or destination IP addresses in the packet headers during routing.

- This enables correct data transmission between the LAN and the internet.

## Controversies Surrounding NAT:

- NAT is controversial because it only processes data up to Layer 3 (the network layer) of the OSI model.

- There are concerns about the address shortage in IPv4, which is expected to be solved by transitioning to IPv6.

- NAT can violate the end-to-end argument, which states that all devices should communicate directly over the internet.

## NAT Traversal:

This is a technique used to connect to servers behind a NAT.

- If a device needs to connect to a server that is behind NAT, special methods (like STUN or TURN) help establish this connection.

## NAT is Here to Stay:

- Despite its controversies, NAT is widely used and will continue to be a standard practice in networking for the foreseeable future.